43008: Reinforcement Learning

Assignment 3, Part-C

Initial Experimental Results

Project Title and number (Refer to Dashboard): Autonomous drone hoop flying with reinforcement learning (DJI Tello) [TBC: project number from the dashboard]

Team Name: [TBC: fill in from the dashboard]

Team Members:

| Student Name | Student ID | Email ID |
| --- | --- | --- |

Algorithms/Methods Chose Type:  Deep reinforcement learning (on-policy actor-critic, PPO), combined with imitation learning

•  Proximal Policy Optimisation (PPO), Stable-Baselines3, the main learner in every stage. A clipped policy-gradient actor-critic with GAE; we add an early-stop on KL divergence (target_kl) as a trust-region brake, which fixed the policy collapse we saw with default settings.

•  Behaviour cloning (BC) from a scripted visual-servo expert and human keyboard flights, used as a warm start.

•  DAgger with a privileged teacher: a controller that can see the true simulator state relabels the states visited by the camera-only student, so the student learns to recover from its own mistakes.

•  DAPG (demo-augmented policy gradient): PPO with a constant imitation term (coefficient 0.3) on the DAgger labels.

•  Asymmetric actor-critic with memory: the actor sees a history of the last ~1 s of camera/telemetry features (lags 0, 1, 2, 4, 8, 16 ticks) and a ring tracker; the critic additionally gets 20 privileged simulator numbers. This was the change that made the drone able to loop.

•  Also available for comparison in our dashboard: A2C, SAC, TD3 (continuous actions) and DQN through a discrete-action wrapper.

Experimental settings:

Problem formulation (MDP), onboard-camera task

| Element | Setting |
| --- | --- |
| State / observation | 41 features per frame: ring detection (found, centre offset, apparent size, ellipse axis ratio, range, age of detection, where it was last seen), forward distance since last seen, heading at last pass, pass streak, height, velocity, attitude, last 4 actions, plus a 4-number ring tracker. Stacked over 6 lags → 246 actor inputs; critic gets +20 privileged sim values |
| Action | 4 continuous values in [-1, 1]: left/right, forward/back, up/down, yaw → Tello rc sticks, 15 Hz |
| Reward | +20 per pass through the ring (same side, with a streak multiplier); −10 hitting the ring; −10 leaving the cage; −5 missing; −20 going back through the wrong way; potential-based shaping towards the ring axis (weight 3, axis pull 1); small rim-clearance term (0.3, margin 0.08 m); time cost 0.02 |
| Episode | 90 s; starts hovering at about 0.8 m from one of 9 cage zones in turn; 60 % slanted starts near the ring and 30 % starts just before a pass during training |
| Randomisation | Drone dynamics (simulator profile fitted to real flight logs), ring position and swing (2°), prop-wash push (15°), detector noise and lag |

Algorithm settings (best run w10_mg6_C9_loop_practice)

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| Algorithm | PPO (SB3) + DAPG term | Learning rate | 5e-5 |
| γ (discount) | 0.99 | GAE λ | 0.95 |
| n_steps per env | 2048 | Batch size | 512 |
| Epochs per update | 5 | target_kl | 0.01 |
| Entropy coefficient | 0.0 | Clip range | 0.2 (SB3 default) |
| Parallel envs | 4 | Imitation (BC) coef. | 0.3, kept constant |
| Demos | 14 DAgger files, ~442k teacher labels | Total steps | 13.7 M (continued from 11.7 M) |
| Evaluation | 36 episodes, 9 zones × 4, deterministic | Hardware | Laptop CPU only, no GPU |

Earlier stages: hover used PPO on HoverAviary (one-dimensional RPM action, 4 envs, early stop at reward 474). The full-state PyBullet hoop task used PPO with lr 1e-4, target_kl 0.03, γ 0.995, batch 256, 5 epochs, 256 × 256 networks, 6 envs, and a wind / ring-size curriculum.

Initial Results:

Table 1. Stage 1: hover (PPO, HoverAviary)

| Training steps | 8k | 16k | 40k | 104k | 152k | 160k (early stop) |
| --- | --- | --- | --- | --- | --- | --- |
| Mean reward (max 474) | 336 | 441 | 219 | 466 | 474 | 474.1 ± 0.0 |

Solved in 160k steps (about 6.5 min on CPU); the drone climbs to 1.0 m and holds it with millimetre-scale drift.

Table 2. Stage 2: reach a randomly placed hoop (PyBullet, 20 evaluation episodes)

| Action space | Steps | Reached the hoop | Mean final distance |
| --- | --- | --- | --- |
| Position target (pid) | 400k | 0 / 20 | 0.84 ± 0.34 m |
| Velocity command (vel) | 500k | 9 / 20 | 0.34 ± 0.22 m |

With position targets the drone tipped over within about 8 control steps every episode, so it never learned; bounded velocity commands fixed it.

Table 3. Stage 3: fly through the ring, full-state PyBullet environment (MDP iterations)

| MDP version | Steps | Eval success (≥ 1 pass) | Best passes / episode | What we learned |
| --- | --- | --- | --- | --- |
| v1 shaped reward, one pass ends episode | 12.2 M | peak 30 %, ~6 % average | — | PPO unstable (approx_kl ≈ 0.9); added KL brake |
| v2 potential shaping, rim −3 | 14.5 M | 0 % | 0 | Agent parked 18–31 cm in front of the ring (reward loophole) |
| v3 time budget, passes don't end episode | 38.0 M | 0–40 % | 0.7 | Never a second pass |
| v4 'armed' flag, hover start, prop-wash ring | 39.1 M | up to 70 % | 0.7 | Mostly from one corner |
| v5 same-side laps, streak bonus, 9 start zones | 59.8 M | up to 100 % | 1.06 | Passes from every start zone |

Table 4. Stage 4: onboard-camera task, laps in 90 s (36 evaluation episodes, sim)

| Policy / method | Laps / ep | ≥ 1 pass | ≥ 2 passes | Oblique entry |
| --- | --- | --- | --- | --- |
| PPO, no memory, ring tracker (T) | 0.39 | 39 % | 0 % | 20 % |
| PPO, no imitation term (W) | 0.67 | 67 % | 0 % | — |
| + memory + asymmetric critic + DAPG (n8) | 1.28 | 83 % | 44 % | — |
| + slanted starts (n9 O) | 1.58 | 81 % | 53 % | 67 % |
| + loop practice starts, DAgger round (mg3 C9) | 2.19 | 89 % | 64 % | 60 % |
| Best: mg6 C9 (final) | 2.33 | 81 % | 64 % | 57 % |

Best single episode: 5 passes. Oblique entry = share of 30 slanted starts near the ring (35–80° off its axis) that end in a clean pass.

Table 5. What did not help (sim)

| Tried | Result | Likely reason |
| --- | --- | --- |
| γ 0.995, bigger streak bonus | worse | not the bottleneck |
| Starts just before the ring, without memory | no better | policy cannot remember the ring behind it |
| Teacher labels for a memoryless policy | collapsed to ~0 laps | 'just passed' and 'about to pass' look identical without memory |
| Imitating human demos | stopped the loop | human flights never loop |

Table 6. Imitation baseline and first real flights

| Test | Result |
| --- | --- |
| Scripted expert, sim (typical drone profile) | 100 % (50/50) passes |
| Behaviour clone of the expert, with the real OpenCV detector | 6 / 12 |
| Memory policy n8_cont2 on the real Tello, bottom-right corner | entered 2 of 3 attempts |
| Same policy, other three corners | found the ring but clipped its edge going in |
| Lighting | night flights worked; daytime flights locked onto the window → per-lighting camera presets |

Next steps: improve slanted (corner) entries, which are the main sim-to-real failure, with more delay/drift randomisation and a line-up reward, then fly the best policy (2.33 laps/ep) on the real drone.